RTC BASED TIME OPERATED APPLIANCES                          CONTROL

INTRODUCTION

Power Saving Using Time Operated Electrical Appliance Controlling System is a reliable circuit that takes over the task of switch on/off the electrical devices with respect to time. This project replaces the Manual Switching. It has an Inbuilt Real Time Clock which tracks over the Real Time. When this time equals to the programmed time, then the corresponding Relay for the device is switched ON. The switching time can be edited at any Time using the keypad. The Real Time Clock is displayed on LCD display.

BLOCK DIAGRAM

8051

RTC

Relay

Relay

Relay

PORT1

PORT 2

PORT3

PORT  0

A BRIEF INTRODUCTION TO 8051 MICROCONTROLLER:

When we have to learn about a new computer we have to familiarize about the machine capability we are using, and we can do it by studying the internal hardware design (devices architecture), and also to know about the size, number and the size of the registers.

A microcontroller is a single chip that contains the processor (the CPU), non-volatile memory for the program (ROM or flash), volatile memory for input and output (RAM), a clock and an I/O control unit. Also called a "computer on a chip," billions of microcontroller units (MCUs) are embedded each year in a myriad of products from toys to appliances to automobiles. For example, a single vehicle can use 70 or more microcontrollers. The following picture describes a general block diagram of microcontroller.

89s52:   The AT89S52 is a low-power, high-performance CMOS 8-bit microcontroller with 8K bytes of in-system programmable Flash memory. The device is manufactured using Atmel’s high-density nonvolatile memory technology and is compatible with the industry-standard 80C51 instruction set and pinout. The on-chip Flash allows the program memory to be reprogrammed in-system or by a conventional nonvolatile memory pro-grammer. By combining a versatile 8-bit CPU with in-system programmable Flash on a monolithic chip, the Atmel AT89S52 is a powerful microcontroller, which provides a highly flexible and cost-effective solution to many, embedded control applications. The AT89S52 provides the following standard features: 8K bytes of Flash, 256 bytes of RAM, 32 I/O lines, Watchdog timer, two data pointers, three 16-bit timer/counters, a six-vector two-level interrupt architecture, a full duplex serial port, on-chip oscillator, and clock circuitry. In addition, the AT89S52 is designed with static logic for operation down to zero frequency and supports two software selectable power saving modes. The Idle Mode stops the CPU while allowing the RAM, timer/counters, serial port, and interrupt system to continue functioning. The Power-down mode saves the RAM con-tents but freezes the oscillator, disabling all other chip functions until the next interrupt

The hardware is driven by a set of program instructions, or software. Once familiar with hardware and software, the user can then apply the microcontroller to the problems easily.

The pin diagram of the 8051 shows all of the input/output pins unique to microcontrollers:

The following are some of the capabilities of 8051 microcontroller.

- Internal ROM and RAM

- I/O ports with programmable pins

- Timers and counters

- Serial data communication

The 8051 architecture consists of these specific features:

    - 16 bit PC &data pointer (DPTR)

    - 8 bit program status word (PSW)

    - 8 bit stack pointer (SP)

    - Internal ROM 4k

    - Internal RAM of 128 bytes.

    - 4 register banks, each containing 8 registers

    - 80 bits of general purpose data memory

    - 32 input/output pins arranged as four 8 bit ports: P0-P3

    - Two 16 bit timer/counters: T0-T1

    - Two external and three internal interrupt sources Oscillator and    clock circuits.