AUTOMATIC CONTROL FOR UNMANNED RAILWAY GATE

The cheapest way to remotely control a device within a visible range is via Infra-Red light. Almost all audio and video equipment can be controlled this way nowadays. Due to this wide spread use the required components are quite cheap, thus making it ideal for us to use IR control for our own projects. Infra-Red actually is normal light with a particular color. We humans can't see this color because its wave length of 950nm is below the visible spectrum. That's one of the reasons why IR is chosen for remote control purposes, we want to use it but we're not interested in seeing it. Another reason is because IR LED’s are quite easy to make, and therefore can be very cheap.

Automatic Control For UnManned Railway Gate projects main idea is to develop a application where railway gates will be controlled automatically using microcontroller system. Implementing this system can save lot of efficiency in controlling railway gates. This application uses 89s52 microcontroller and IR transmitters and receivers.

In present system railway gates are maintained manually, but there are many cases where human errors which had caused huge damages. In order to develop this application accuracy is most important so there is need to be a quality check before implementing in reality.

In this project two powerful transmitters and receivers are used, among them one transmitter and one receiver is used at the upside at the human level and other pair is fixed at downside.

Present project is designed using 89S52 microcontroller to avoid railway accidents happening at unattended railway gates, if implemented in spirit. This project utilizes two powerful IR transmitters and two receivers; one pair of transmitter and receiver is fixed at up side (from where the train comes) at a level higher than a human being in exact alignment and similarly the other pair is fixed at down side of the train direction. Sensor activation time is so adjusted by calculating the time taken at a certain speed to cross at least one compartment of standard minimum size of the Indian railway. We have considered 5 seconds for this project. Sensors are fixed at 1km on both sides of the gate. We call the sensor along the train direction as ‘foreside sensor’ and the other as ‘aft side sensor’. When foreside receiver gets activated, the gate motor is turned on in one direction and the gate is closed and stays closed until the train crosses the gate and reaches aft side sensors. When aft side receiver gets activated motor turns in opposite direction and gate opens and motor stops. Buzzer will immediately sound at the fore side receiver activation and gate will close after 5 seconds, so giving time to drivers to clear gate area in order to avoid trapping between the gates and stop sound after the train has crossed.

The same principle is applied for track switching. Considering a situation wherein an express train and a local train are traveling in opposite directions on the same track; the express train is allowed to travel on the same track and the local train has to switch on to the other track. Two sensors are placed at the either sides of the junction where the track switches. If there’s a train approaching from the other side, then another sensor placed along that direction gets activated and will send an interrupt to the controller. The interrupt service routine switches the track.  Indicator lights have been provided to avoid collisions. Here the switching operation is performed using a DC motor. Assuming that within a certain delay, the train has passed the track is switched back to its original position, allowing the first train to pass without any interruption. This concept of track switching can be applied at 1km distance from the stations.